Enterprise networking Project #5

Radeon Company Ltd. is a US-owned company that deals with Banking and Insurance. The company is intending to expand its services across the African continent having the first branch to be located in Nairobi, Kenya. The company has secured a four-story building to operate within the Kenyan capital city. Therefore, the company would like to allow sourcing the knowledge from a group of final-year students from the local university to design and implement their company network. Assume you are among the students to take over this role, carefully read down the requirements then model the design and implement the network based on the company's needs. Each floor has departments as provided in the table below.

First Floor

No.       Departments              No of Pc           No of printers

1 	  Management                  20                  4

2         Research 	               20	          4

3         Human Resources    		20                4

Second floor

No.		Departments		No of Pc 		No of printers

1		Marketing 		20				4

2		Accounting 		20				4

3		Finance 		20				4

Third Floor

No.		Departments				No of Pc 			No of printers

1		Logistics and store 			20					4

2		Customer care 				20					4

3		Guest area 				20					2

Fourth floor

No.		Departments			No of Pc 			No of printers 			No of servers

1		Administration 			20					2

2		ICT 				20					2

3 		Server room			2 Admin PCs 							3(Dhcp,Http,email)

Requirements:

Curutech Solutions

1. Use a software modeling tool to visualize the network topology (consider requirement 3)

Software Modelling Tools: MS Visio, Visual Paradigm, or Draw.io for modeling network design.

2. Use any of the following network simulation software to implement the above topology:

Simulation software: Cisco Packet tracer or GNS3 for design and implementation.

There should be one router on each floor. The router should be connecting switches on that floor.

Use OSPF as the routing protocol to advertise routes.

Each department is required to have a wireless network for the users.

Each department except the server room will be anticipated to have around 60 users both

wired and wireless users.

Host devices in the network are required to obtain IPv4 addresses automatically.

Devices in all the departments are required to communicate with each other.

All devices in the network are expected to obtain an IP address dynamically from the dedicated DHCP servers located at the server room.

Create HTTP. and E-mail servers

Configure SSH in all the routers for remote login.

3. Use hierarchical network design with redundancy included:

Having core, distribution, and access layers.

4. Configure the basic configuration of the devices:

Hostnames

Line Console and VTY passwords

Banner messages

Disable domain IP lookup

5. Each department should be in a different VLAN

Create VLANs in every department

VLANs you will use in your case, including VLANI also e.g. 10, 20, 30... etc.

Each VLAN should be a different subnetwork.

6. Planning of IP Addresses:

You have been given 192.168.10.0 as the base address for this network.

AND 10.10.10.10.0 for the routing

Do subnetting based on the number of hosts in every department as provided above.

Identify subnet mask, useable IP address range, and broadcast address for each subnet.

7. End Device Configurations:

Configure all the end devices in the network with the appropriate IP address based on the calculations above.

9. Test Communication:

Do devices in the same VLAN communicate?

Do the devices in different VLANs communicate?

10. Document the project design and implementation